This document is intended as a suggested guide for creating, modifying, or editing your 3-Part Architectural Guide Specifications.

Tarkett will not be liable for any damages arising out of the use of any information or specifications found in this document.

Ensure that you have the latest publication for this specification.

The specifier or designer is responsible for product selection and accuracy of all project specifications, including any Tarkett information or specifications used.

GENERAL

RELATED DOCUMENTS

Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

SUMMARY

Section Includes:

Resilient Wall Base.

SUBMITTALS

Product Data:  For each type of product indicated.

LEED Submittals:

Product Data for Credit EQ 4.1:  For adhesives, include printed statement of VOC content and chemical components.

Samples for Initial Selection:  For each type of product indicated.

Samples for Verification:  For each type of product indicated, in manufacturer's standard-size samples of each resilient product color, texture, and pattern required.

Product Schedule:  For resilient products. Use same designations indicated on Drawings.

QUALITY ASSURANCE

Installation Qualification: Contractors for floor covering installation should be experienced in managing commercial flooring projects and provide professional installers, qualified to install the various flooring materials specified. An installer is “qualified” if trained, or a certified by Tarkett or a certified INSTALL (International Standards & Training Alliance) resilient floor covering installer.

Mockups:  Provide resilient products with mockups specified in other Sections.

DELIVERY, STORAGE, AND HANDLING

Store resilient products and installation materials in dry spaces protected from the weather, with ambient temperatures maintained within range recommended by Tarkett, but not less than 55 deg F (13 deg C) or more than 85 deg F (29 deg C).

PROJECT CONDITIONS

Install resilient products after other finishing operations, including painting, have been completed.

Maintain ambient temperatures within range recommended by Tarkett, but not less than 65 deg F (18 deg C) or more than 85 deg F (29 deg C) in spaces to receive resilient products during the following time periods:

48 hours before installation.

During installation.

48 hours after installation.

Maintain the ambient relative humidity between 40% and 60% during installation.

Until Substantial Completion, maintain ambient temperatures within range recommended by Tarkett, but not less than 55 deg F (13 deg C) or more than 85 deg F (29 deg C).

PRODUCTS

RESILIENT SHEET FLOORING

| Manufacturer: |  |  |
| --- | --- | --- |
| Tarkett North America | Phone: | (800) 899-8916 |
| 30000 Aurora Rd. |  |  |
| Solon, Ohio 44139 |  |  |
| Web: www.tarkettna.com |  |  |
| E-mail: info@tarkett.com |  |  |

PERFORMANCE REQUIREMENTS

Fire-Test-Response Characteristics: For resilient tile flooring, as determined by testing identical products according to ASTM E 648 or NFPA 253 by a qualified testing agency.

Critical Radiant Flux Classification: Class I, not less than 0.45 W/sq. cm.

Flooring products shall comply with the requirements of the California Department of Public Health's "Standard Method for the Testing and Evaluation of Volatile Organic Chemical Emissions from Indoor Sources Using Environmental Chambers."

RESILIENT TRADITIONAL RUBBER DURACOVE WALL BASE

Basis-of-Design Product: Subject to compliance with requirements, provide Tarkett Traditional Thermoplastic Rubber Wall Base.

Performance requirements meets ASTM F1861 Standard Specification for Resilient Thermoplastic Rubber Wall Base, Type TP, Group 1.

For thickness specify, 0.125" (3.17 mm)

For type, specify: [Straight] [Coved] <Insert type>

For height specify: [2.5" (6.35 cm)], [4" (10.16 cm)], [4.5" (11.4 cm)] or [6" (15.24 cm)] <Insert height>

For 2.5", 4" or 4.5" heights, specify length: [4' (1.22 m)] [120' (36.58 m)] <Insert length>

For 6" heights, specify length: [4' (1.22 m)] [100' (30.48 m)] <Insert length>

For corners, specify: [Inside Corners] [Outside Corners]

Colors and Patterns: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from full range of industry colors] <Insert colors and patterns>.

Test Data:

Flexibility, ASTM F137: Passes 1/4 inch mandrel

Resistance to light, ASTM F1515: Passes

Resistance to chemicals, ASTM F925: Passes

ASTM E 648, Standard Test Method for Critical Radiant Flux of 0.45 watts/cm2 or greater, Class 1.

RESILIENT TRADITIONAL VINYL WALL BASE

Basis-of-Design Product: Subject to compliance with requirements, provide Tarkett Traditional Thermoplastic Vinyl Wall Base.

Performance requirements meets ASTM F1861 Standard Specification for Resilient Vinyl Wall Base, Type TV, Group 1.

For thickness, specify: [0.125" (3.17 mm)] [0.080" (2 mm)] <Insert thickness>

For type, specify: [Straight] [Coved] <Insert type>

For height, specify: [2.5" (6.35 cm)], [4" (10.16 cm)] or [6" (15.24 cm)] <Insert height>

For 2.5", 4" or 4.5" heights, specify length: [4' (1.22 m)] [120' (36.58 m)] <Insert length>

For 6" heights, specify length: [4' (1.22 m)] [100' (30.48 m)] <Insert length>

For corners, specify: [Inside Corners] [Outside Corners]

Colors and Patterns: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from full range of industry colors] <Insert colors and patterns>.

Test Data:

Flexibility, ASTM F137: Passes 1/4 inch mandrel

Resistance to light, ASTM F1515: Passes

Resistance to chemicals, ASTM F925: Passes

ASTM E 648, Standard Test Method for Critical Radiant Flux of 0.45 watts/cm2 or greater, Class 1.

RESILIENT PERCEPTIONS RUBBER WALL BASE

Basis-of-Design Product: Subject to compliance with requirements, provide Tarkett; Perceptions [Recess coved] [Recess straight] [Flex] [Contour] [Royale] or comparable product.

Performance requirements meets ASTM F1861 Standard Specification for Resilient Thermoplastic Rubber Wall Base, Type TP, Group 1.

For thickness, specify: [0.125" (3.17 mm)]

For height, specify: [4.25" (10.8 cm)]

Specify length: [120' (36.58 m)]

For corners, specify: [Inside Corners] [Outside Corners]

Colors and Patterns: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from full range of industry colors] <Insert colors and patterns>.

Test Data:

Flexibility, ASTM F137: Passes 1/4 inch mandrel

Resistance to light, ASTM F1515: Passes

Resistance to chemicals, ASTM F925: Passes

ASTM E 648, Standard Test Method for Critical Radiant Flux of 0.45 watts/cm2 or greater, Class 1.

RESILIENT BASEWORKS WALL BASE

Basis-of-Design Product: Subject to compliance with requirements, provide Tarkett Traditional Rubber Wall Base.

Performance requirements meets ASTM F1861 Standard Specification for Resilient Thermoset Rubber Wall Base, Type TS, Group 1.

For thickness, specify, 0.125" (3.17 mm)

For type, specify: [Straight] [Coved] <Insert type>

For height, specify: [2.5" (6.35 cm)], [4" (10.16 cm)], or [6" (15.24 cm)] <Insert height>

For 2.5" and 4" heights, specify length: [4' (1.22 m)] [120' (36.58 m)] <Insert length>

For 6" heights, specify length: [4' (1.22 m)] [100' (30.48 m)] <Insert length>

For corners, specify: [Inside Corners] [Outside Corners] <Insert Corners>

Colors and Patterns: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from full range of industry colors] <Insert colors and patterns>.

Test Data:

Flexibility, ASTM F137: Passes 1/4 inch mandrel

Resistance to light, ASTM F1515: Passes

Resistance to chemicals, ASTM F925: Passes

ASTM E648, Standard Test Method for Critical Radiant Flux of 0.45 watts/cm2 or greater, Class 1.

ASTM E84, Flame Spread and Smoke Development: Class B, ≤ 450

RESILIENT TIGHTLOCK RUBBER WALL BASE

Basis-of-Design Product: Subject to compliance with requirements, provide Tarkett Traditional Rubber Wall Base.

Performance requirements meets ASTM F1861 Standard Specification for Resilient Rubber Wall Base, Type TP, Group 1.

For type, specify: [TightLock for Carpet] or [TightLock for Resilient] <Insert type>

TightLock for Carpet height specify: [3.25" (8.26 cm)], [4.5" (11.43 cm)] or [6.5" (16.51 cm)] <Insert height>

TightLock for Resilient height specify: [3.125" (7.94 cm)], [4.375" (11.11 cm)] or [6.375" (16.2 cm)] <Insert height>

For length specify: [4' (1.22 m)] [75' (22.86 m)] <Insert length>

For corners, specify: [Inside Corners] [Outside Corners]

Colors and Patterns: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from full range of industry colors] <Insert colors and patterns>.

Test Data:

Flexibility, ASTM F137: Passes 1/4 inch mandrel

Resistance to light, ASTM F1515: Passes

Resistance to chemicals, ASTM F925: Passes

ASTM E 648, Standard Test Method for Critical Radiant Flux of 0.45 watts/cm2 or greater, Class 1.

RESILIENT MILLWORK CONTOURABLE WALL BASE

Basis-of-Design Product: Subject to compliance with requirements, provide Johnsonite; A Tarkett Company; Millwork [Attaché] [Ambassador] [Diplomat] [Delineate] [Equinox] [Inflection] [Mandalay] [Monument] [Monarch] [Oblique] [Outline] [Reveal] [Silhouette] or comparable product.

Product Standard: Meets performance requirements for ASTM F 1861 Standard Specification for Resilient Wall Base, Type TP, Group 1.

For Mandalay specify height: [2.5" (6.35 cm)], [3" (7.62 cm)], [4.5" (11.43 cm)] or [6" (15.24 cm)] <Insert height>

For Reveal specify height: [4.25" (10.8 cm)] [6" (15.24 cm)] or [8" (20.32 cm)] <Insert height>

For Monument specify height: [4" (10.16 cm)] or [2.5" (6.35 cm)] <Insert height>

Specify length: 8 feet (2.4 m).

Colors and Patterns: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from full range of industry colors] <Insert colors and patterns>.

Test Data:

Resistance to light, ASTM F1515: Passes

Resistance to chemicals, ASTM F925: Passes

ASTM E 648, Standard Test Method for Critical Radiant Flux of 0.45 watts/cm2 or greater, Class 1.

RESILIENT MASQUERADE WALL BASE

Basis-of-Design Product: Subject to compliance with requirements, provide Johnsonite; A Tarkett Company; Masquerade [Classic] [Oblique] [Outline] [Reveal] [Silhouette] [Traditional Wall Base] or comparable product.

Meets performance requirements for ASTM F 1861 Standard Specification for Resilient Wall Base, Type TP, Group 1.

Specify length for Traditional Wall Base: 4 feet (1.22 m).

Specify length for Classic, Oblique, Outline, Reveal, or Silhouette Wall Base: 8 feet (2.44 m).

Colors and Patterns: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from full range of industry colors] <Insert colors and patterns>.

Test Data:

Resistance to light, ASTM F1515: Passes

Resistance to chemicals, ASTM F925: Passes

ASTM E 648, Standard Test Method for Critical Radiant Flux of 0.45 watts/cm2 or greater, Class 1.

      - INSTALLATION MATERIALS

Adhesives:  as recommended by Tarkett to meet site conditions

Tarkett 960 Cove Base Adhesive (Porous applications)

Tarkett 946 Premium Contact Bond Adhesive (Non-porous applications)

EXECUTION

EXAMINATION

Examine substrates, with Installer present, for compliance with requirements for maximum moisture content and other conditions affecting performance of the work.

Verify that finishes of substrates comply with tolerances and other requirements specified in other Sections and that substrates are free of cracks, ridges, depressions, scale, and foreign deposits that might interfere with adhesion of resilient products.

Proceed with installation only after unsatisfactory conditions have been corrected.

PREPARATION

Prepare substrates according to Tarkett's written instructions to ensure adhesion of resilient wall base.

Fill cracks, holes, and depressions in substrates with trowelable leveling and patching compound and remove bumps and ridges to produce a uniform and smooth substrate.

Move resilient products and installation materials into spaces where they will be installed at least 48 hours in advance of installation.

Vacuum clean substrates to be covered by resilient products immediately before installation.

RESILIENT BASE INSTALLATION

Comply with Tarkett's written instructions for installing resilient base.

Apply resilient base to walls, columns, pilasters, casework and cabinets in toe spaces, and other permanent fixtures in rooms and areas where base is required.

Install resilient base in lengths as long as practicable without gaps at seams and with tops of adjacent pieces aligned.

Tightly adhere resilient wall base to substrate throughout length of each piece, with base in continuous contact with horizontal and vertical substrates.

Do not stretch resilient base during installation.

CLEANING AND PROTECTION

Comply with Tarkett's written instructions for cleaning and protection of resilient products.

Perform the following operations immediately after completing resilient product installation:

Remove adhesive and other blemishes from exposed surfaces.

Protect resilient products from mars, marks, indentations, and other damage from construction operations and placement of equipment and fixtures during remainder of construction period.

END OF SECTION 09.65.13